STEPS TO MAKE SETUP OF NRF5340 ON VS CODE

- Install nrf commad line tool

Link- https://www.nordicsemi.com/Products/Development-tools/nrf-command-line-tools/download

- Install Nrf connect for desktop

Link- https://www.nordicsemi.com/Products/Development-tools/nRF-Connect-for-Desktop/Download#infotabs

- Install VS code.

Link- https://code.visualstudio.com/docs/?dv=win

- Open nrf connect and install “toolchain manager” and click on “Open”.

- Then install the latest available SDK, in my case it was 2.2.0.

- Click on “Open VS Code”

- Click on “install missing extensions”

- Click on “ Open VS Code”

- On welcome window for nrf connect select the following path

- Click on nrf extension and select your desired  example and create application

- You will see blinky code under application head

- Click on “add build configuration”, a new window will come.

- Select your desired board and click “build Configuration”.

- After build configuration you will see the following view on your screen

- Connect your device and click on flash.

- If you make any changes in code don’t forget to build your code first and then flash.

IMP link- https://www.nordicsemi.com/Products/Development-tools/nRF-Connect-for-VS-Code/Tutorials